ecognition of Prior Learning (RPL)

ASSESSMENT GUIDE FOR

OPERATING DIFFERENT VEHICLES

National Certificate in Professional Driving

Qualification ID:  50285

Evidence Guide for Document 6

Operating Different Vehicles

UNIT STANDARDS IN THIS VOLUME

| Unit Standard Number | Unit Standard Title | NQF Level | Credit Value |
| --- | --- | --- | --- |
| 123257 | Operate a rigid light vehicle | 2 | 10 |
| 8038 | Operating lift trucks | 3 | 6 |
| 123253 | Operate a rigid heavy vehicle | 4 | 15 |
| 123254 | Operate a vehicle combination | 4 | 20 |

TABLE OF CONTENTS

| Description | Page Number |
| --- | --- |
| Unit Standard 1 of this Volume |  |
| Specific outcome 1 answers and competency criteria | 5 – 6 |
| Specific outcome 2 answers and competency criteria | 7 – 9 |
| Specific outcome 3 answers and competency criteria | 10 – 11 |
| Specific outcome 4 answers and competency criteria | 12 - 13 |
| Specific outcome 5 answers and competency criteria | 14 – 15 |
| Specific outcome 6 answers and competency criteria | 16 |
| Unit Standard 2 of this Volume |  |
| Specific outcome 1 answers and competency criteria | 17 - 18 |
| Specific outcome 2 answers and competency criteria | 19 |
| Specific outcome 3 answers and competency criteria | 20 – 21 |
| Specific outcome 4 answers and competency criteria | 22 – 23 |
| Specific outcome 5 answers and competency criteria | 24 |
| Unit Standard 3 of this Volume |  |
| Specific outcome 1 answers and competency criteria | 25 – 26 |
| Specific outcome 2 answers and competency criteria | 27 – 28 |
| Specific outcome 3 answers and competency criteria | 29 - 30 |
| Specific outcome 4 answers and competency criteria | 31 – 32 |
| Specific outcome 5 answers and competency criteria | 33 |
| Unit Standard 4 of this Volume |  |
| Specific outcome 1 answers and competency criteria | 34 – 35 |
| Specific outcome 2 answers and competency criteria | 36 - 37 |
| Specific outcome 3 answers and competency criteria | 38 – 39 |
| Specific outcome 4 answers and competency criteria | 40 - 41 |
| Specific outcome 5 answers and competency criteria | 42 |
| Specific outcome 6 answers and competency criteria | 43 |

Unit Standard 1 of this Volume

Answers for Evidence Guide

Unit Standard ID Title:  Operate a rigid light vehicle

Unit Standard number:  123257

Specific Outcome 1

Prepare a rigid light vehicle for road transport trips according to specification.

Assessment Criteria

- Vital rigid light vehicle components are inspected in a systematic way in order to comply with the relevant legislation. (Relevant legislation currently includes the Road Traffic Act of 1993.)

- Rigid light vehicle inspection is recorded legibly and in accordance with operational requirements/specifications.

- All rigid light vehicle defects are identified and reported accurately, comprehensively and timeously, so that the necessary actions to rectify defects are initiated.

- All prescribed and or other relevant preliminaries are executed in terms legislative and organisational requirements. (Preliminaries can include brake tests and start up procedures.)

| Evidence Required Written Knowledge Test |  | Evidence sign off |
| --- | --- | --- |
| Question 1 (3) Name the different license codes next to each specification that drivers need to operate. - Rigid vehicle above 16000 kg ( C ) - Light motor vehicle up to 3500 kg with a trailer of 900 kg (EB) - Combination or articulated vehicle above 16000 kg (EC) Question 2 (2) Describe the reason to prepare a vehicle for a trip. To identify and defects and to ensure the vehicle is safe to use on the roads and no risk to others. Question 3 (4) List the four main aspects of preparing the vehicle for a trip. - Inspect the vital components of the vehicle in a systematic manner - Record which components are serviceable - Record and report any components that are defective - Conduct a brake and steering test Question 4 (3) List three types of fluids that must be checked on a vehicle. - Engine oil - Coolant - Brake fluid Question 5 (2) Name two preliminary tests’ a driver should perform before leaving the depot. Steering test ad brake test |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

In order to declare the learner competent he/she must obtain at least 7/14 for the test but does not apply for the practical evidence below.

| Evidence Required Inspection list (Prepare a rigid light vehicle for road transport trips according to specification) |  | Evidence sign off |
| --- | --- | --- |
| The learner must use an appropriate vehicle inspection list. Did the learner check the following …(if applicable) Criteria for inspection; Yes; No. 1; Vehicle for any damage (#). 2; Vehicle for cleanliness. 3; The windscreen for damage and cleanliness (#). 4; The license disc is still valid (#). 5; The windscreen wipers condition and wipe clean. 6; The head light, indicator lenses & front reflectors (#). 7; The registration plate. 8; The side mirrors (#). 9; The doors & windows condition & company logo. 10; The front right hand side tyre, rim, wheel nuts, valve (#). 11; The battery (#). 12; The exhaust system (#). 13; The condition of the vehicle body (#). 14; The spare wheel (#). 15; The right hand side rear types, rims, wheel nuts and valves (#). 16; The tail & stop light lenses & the rear indicators lenses. 17; The rear registration plate. 18; The left hand side rear tyres, rims, wheel nuts and valves (#). 19; The fuel tank and cap. 20; The front left hand side tyre, rim, wheel nuts, valve. 21; Engine oil level. 22; Coolant level (#). 23; Brake fluid level (#). 24; Clutch fluid level (#). 25; Power steering fluid level. 26; Fan belt condition (#). 27; All engine components are secure. 28; The vehicle jack and wheel spanners are present and secure (#). 29; The vehicles emergency triangles (#). 30; The vehicle fire extinguisher (#). 31; Doors & windows operate correctly. 32; Adjust seat accordingly (#). 33; The cab floor is free of loose obstructions (#). 34; The rear view mirror (#). 35; Vehicle park brake is applied. 36; Switch on ignition and check all instruments are working (#). 37; The hooter , wipers, lights and indicators are working (#). |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

In order to declare the learner competent he/she must meet all the crucial outcomes and specified criteria according to this context.  See the … (#) symbol that indicates that the outcome is a prerequisite to be declared competent.  If the learner do not comply with these specifications then it is crucial for the learner to master these outcomes because this can have crucial effects and impact.  The learner is declared not yet competent after providing him or her with two chances to master the outcomes.  The assessor must use his/her own discretion based on his/her practical experience.

Answers for Evidence Guide

Specific Outcome 2

Drive a rigid light vehicle in accordance with specified requirements.

Outcome Range:

Specified requirements include legal, manufacturer, and defensive driving requirements; current legal requirements include K53.

Assessment Criteria

- Rigid light vehicle is driven and manoeuvred in accordance with the specified standard Range: K53.

- Rigid light vehicle is operated in accordance with manufacturer’s specifications.

- Rigid light vehicle is driven and manoeuvred in accordance with legal provisions. (The Road Traffic Act of 1993.)

- Rigid light vehicle is driven to actively prevent accidents, injury to people or damage to property/vehicle, despite the incorrect actions of others or adverse conditions, by driving safely and in a defensive manner.

| Evidence Required Written Knowledge test |  | Evidence sign off |
| --- | --- | --- |
| Question 1 (2) Name two types of hazards. - Stationary hazards - Moving hazards Question 2 Write the full words for the following abbreviations: - SIPDE (5) - Search - Identify - Predict - Decide - Execute - ABC (3) - Attitude - Back-off - Clear space Question 3 (1) How many seconds should there be between your vehicle and the vehicle in front? Three Question 4 (3) List three emergencies a driver may have to deal with. - Incidents - Accidents - Medical Question 5 (2) List two types of possible incidents. - Skidding - Running of the road Question 6 (4) List the four actions of a driver when he/she is involved in an accident, according to the Road Traffic Act. - Stop - Make the situation safe for others - Render first aid where possible - Send or call for assistance Question 7 List five possible mechanical failures that may occur. (5) - Engine overheating - Battery charging light stays on - Loss of brake pressure - Punctures - Fan belt breaking Question 8 Describe ‘K53’. (1) It is the Testing standard for Drivers licenses Total 26 |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

The learner must obtain a minimum of 13/26 in order to be declared competent for the test but does not apply for the practical evidence below.

Criteria for Assessment

The Assessor will explain to the learner what route is required. Make an appointment when you are ready for assessment on the practical demonstration based on the criteria below.

The assessor may ask the learner to drive a specific route according to the K53 system of vehicle control and within the vehicle Manufacturers specifications.

ANY ONE OF THESE  (O) MARKED MEANS THE LEARNER IS NOT YET COMPETENT

|  | USE OF CONTROLS |  | STRENGTHS |  | REMARKS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MANIPULATION OF CONTROLS |  |  |  |  |  |
| 1 |  | Excessive movement |  | ⁮⁮ O |  |  |  |
|  |  | USE OF GEARS |  |  |  |  |  |
| 2 |  | Changes excessively |  | ⁮⁮⁮⁮ O |  |  |  |
| 3 |  | Grates gears |  | ⁮⁮⁮⁮ O |  |  |  |
| 4 |  | Hand rests on top of gear lever |  | ⁮⁮⁮ O |  |  |  |
| 5 |  | Misses or slips a gear |  | O |  |  |  |
|  |  | USE OF CLUTCH |  |  |  |  |  |
| 6 |  | Jerky |  | ⁮⁮⁮ O |  |  |  |
| 7 |  | Slips clutch |  | ⁮ O |  |  |  |
| 8 |  | Rides clutch |  | ⁮⁮ O |  |  |  |
| 9 |  | Late in de-clutching |  | ⁮⁮⁮⁮ O |  |  |  |
| 10 |  | Keeps clutch depressed while stopped |  | ⁮⁮O |  |  |  |
|  |  | USE OF BRAKES |  |  |  |  |  |
| 11 |  | Does not brake progressively |  | ⁮⁮O |  |  |  |
| 12 |  | Park brake not used correctly |  | ⁮⁮ O |  |  |  |
| 13 |  | Brake used unnecessarily (no deceleration) |  | ⁮⁮⁮ O |  |  |  |
| 14 |  | Incorrect use on decline |  | ⁮ O |  |  |  |
| 15 |  | Rests foot on brake pedal |  | ⁮⁮ O |  |  |  |
|  |  | PEDAL BALANCE |  |  |  |  |  |
| 16 |  | Rolling back during pull off |  | ⁮⁮O |  |  |  |
|  |  | STEERING |  |  |  |  |  |
| 17 |  | Crossing arms while turning |  | ⁮⁮⁮⁮ O |  |  |  |
| 18 |  | Hands on steering wheel position |  | ⁮⁮⁮ O |  |  |  |
| 19 |  | Steers with one hand |  | ⁮ O |  |  |  |
|  |  | ACCELERATOR MANAGEMENT |  |  |  |  |  |
| 20 |  | Does not maintain constant speed |  | ⁮⁮ O |  |  |  |
| 21 |  | Blipping |  | ⁮⁮ O |  |  |  |
| 22 |  | Lets engine labour |  | ⁮⁮ O |  |  |  |
| 23 |  | Over-revs |  | ⁮⁮ O |  |  |  |
|  |  | USE OF RETARDATION DEVICE |  |  |  |  |  |
| 24 |  | Not according to specification |  | ⁮⁮ O |  |  |  |
|  |  | COASTING |  |  |  |  |  |
| 25 |  | De-clutch too early |  | ⁮⁮⁮ O |  |  |  |
| 26 |  | Select neutral too early |  | ⁮⁮ O |  |  |  |
|  |  | ROAD SENSE |  |  |  |  |  |
|  |  | ROAD OBSERVATION |  |  |  |  |  |
| 27 |  | No observation |  | ⁮⁮ O |  |  |  |
| 28 |  | Ignores changes in road surface |  | ⁮⁮⁮ O |  |  |  |
| 29 |  | Ignores road signs & markings |  | ⁮⁮ O |  |  |  |
| 30 |  | Moves into other vehicles blind spots |  | ⁮⁮ O |  |  |  |
| 31 |  | Incorrect use of mirrors |  | ⁮⁮⁮ O |  |  |  |
|  |  | VEHICLE POSITIONING |  |  |  |  |  |
| 32 |  | Travels to close to vehicle in front |  | ⁮⁮ O |  |  |  |
| 33 |  | Travel too far from vehicle in front |  | ⁮⁮⁮ O |  |  |  |
| 34 |  | Travels too far over in lane |  | ⁮⁮⁮⁮ O |  |  |  |
|  |  | SPEED |  |  |  |  |  |
| 35 |  | Ignores speed limit |  | ⁮ O |  |  |  |
| 36 |  | Takes too long to pull off at traffic lights, etc. |  |  |  |  |  |
|  |  | ASSESSMENT OF HAZARDS |  |  |  |  |  |
| 37 |  | Fails to notice hazard |  | ⁮⁮ O |  |  |  |
| 38 |  | Fails to react to hazard |  | ⁮⁮ O |  |  |  |
|  |  | CORNERING |  |  |  |  |  |
| 39 |  | Cuts corner |  | ⁮⁮ O |  |  |  |
| 40 |  | Too fast |  | ⁮ O |  |  |  |
| 41 |  | Mounts curb |  | ⁮ O |  |  |  |
| 42 |  | Too wide |  | ⁮ O |  |  |  |
|  |  | OVERTAKING |  |  |  |  |  |
| 43 |  | Fails to check safety ahead |  | O |  |  |  |
| 44 |  | Fails to check safety behind |  | O |  |  |  |
| 45 |  | Fails to back down |  | ⁮ O |  |  |  |
| 46 |  | Accelerates when being overtaken |  | ⁮ O |  |  |  |
|  |  | COMMUNICATION AND ATTITUDE |  |  |  |  |  |
| 47 |  | Hooter used when not required |  | ⁮ O |  |  |  |
| 48 |  | Indicators not used correctly |  | ⁮⁮⁮ O |  |  |  |
| 49 |  | Shows irritation of other road users |  | ⁮ O |  |  |  |
| 50 |  | Demonstrates displeasure to others |  | O |  |  |  |
|  |  | MANOEUVERING |  |  |  |  |  |
| 51 |  | Alley docking from left |  | ⁮⁮⁮⁮ O |  |  |  |
| 52 |  | Alley docking from right |  | ⁮⁮⁮⁮ O |  |  |  |
| 53 |  | Reversing in a straight line |  | ⁮⁮⁮ O |  |  |  |

Assessment Criteria for Competency:

In order to declare the learner competent he/she must meet all the specifications in the practical demonstration assessment.  The assessor must use his/her own discretion based on his/her practical experience.

Answers for Evidence Guide

Specific Outcome 3

Ensure the maintenance of road transport service quality.

Assessment Criteria

- Time schedules are considered, implications for freight/passengers are established, and appropriate action is taken to ensure the maintenance of freight quality/passenger safety and comfort.

- The impact of load positioning on the stability of the rigid light vehicle is accurately described.

- Driving style is adapted in order to maintain freight quality/passenger safety and comfort during transportation

- Organisational standards are adhered to with full consideration of safety and comfort factors and appropriate actions implemented to rectify problems are appropriate for specific contexts. (Safety factors can include stopping only at scheduled locations, passenger door only opened and closed when completely stationary, load (e.g., number of passengers, freight mass, loading of coach and trailer, etc.) restricted to design and legal limit of vehicle, luggage and emergency doors fully secured, attachment of luggage trailer confirmed; Comfort factors include acceleration and deceleration forces applied gradually; cornering speeds maintained within comfort levels; moving off only when passengers are seated or standing securely.)

| Evidence Required Practical Demonstration |  | Evidence sign off |
| --- | --- | --- |
| The learner must include the following evidence in the Portfolio of Evidence. Criteria; Managing unforeseen problems; Yes; No. Time schedules; The learner communicated any implications relating to freight or passengers. The learner has taken appropriate action to the maintenance of fright quality and or ensured the passengers safety in line with the safety procedures. The learner is able to describe the impact of load positioning on the stability of the rigid light vehicle; The learner is able to illustrate how to resolve a problem when the freight is not stable. Practical Demonstration: The learner is able to demonstrate …; Managing unforeseen problems; Yes; No. That the driving style is adapted in order to maintain freight handling during transportation; The learner is able to demonstrate how he/she will act if there are unforeseen problems e.g. weather, rain etc.. Organisational standards are adhered to with full consideration of safety and comfort factors and appropriate actions implemented to rectify problems are appropriate for specific contexts. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

In order to declare the learner competent he/she must meet all the specifications in the practical demonstration assessment.  The assessor must use his/her own discretion based on his/her practical experience.

Answers for Evidence Guide

Specific Outcome 4 
Handle unexpected situations according to specified procedures.

Outcome Range:

Unexpected situations can include, but are not limited to incidents, accidents, breakdowns, fire on vehicle, medical emergency, hi-jacking, mechanical failure (e.g. burst tyre), spillage/load loss, etc.

Assessment Criteria

- Rigid light vehicle and cab instruments are monitored for malfunctions and appropriate action is taken, where required.

- Driving is adapted to unexpected situations safely and according to specified procedures.

- Unexpected situations are reported according to operational procedures and legal requirements.

- Corrective actions to get freight/passengers to their destinations in the event of route deviations, delays experienced and breakdowns, as well as the company guidelines for consideration in selecting course of action described for specific contexts.

| Evidence Required Practical Questioning and Illustration The learner must explain how he/she will adapt to the following situations |  | Evidence sign off |
| --- | --- | --- |
| Section A: Practical Practical Questioning and Illustration (Strictly use the Health and Safety Procedures and other standards in this context) The learner must be able to demonstrate action and explain the steps to follow with the following unforeseen problems that may occur: (Mark a tick next to each answer that was provided and where learner is found competent) - Accidents Fire on vehicle - Medical emergency Hi-jacking - Mechanical failure - Spillage Potholes - Bad weather conditions 21 Marks Written Knowledge Test Question 1 (1) What must a hijacker do to hijack your vehicle? Must stop it and be close enough to get to the driver or passenger Question 2 (6) What should a driver do if he / she is hijacked? - Stay calm - Do what they say and give them what they ask for - Do not put your hands up unless they say so - Do not put your hands up unless they say so - Do not move suddenly or try to get something from your pocket unless they say you must - Call for assistance as soon as possible after the hijack Question 3 (3) List three additional aspects a driver must consider apart from driving the vehicle. - Road conditions - Traffic conditions - Driver fatigue Question 4 (3) List three possible causes of a skid. - Excessive speed - Harsh braking - Harsh acceleration Total: 13 Problem solving question (In line with company procedures) (8) Question 5 Explain the procedures to follow when you are involved in an accident where a pedestrian has been killed … The learner stated the following criteria: Criteria; Yes; No. 1; Stop the vehicle immediately where it is?. 2; Make the accident scene safe to prevent other accidents. 3; Ask someone to call for help. 4; Render any applicable first aid that he/she is trained for. 5; Take down victims and witnesses details. 6; Draw a sketch of the scene. 7; Report the accident to the police within 24 hours. 8; Apply the company specific procedures. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

The learner must obtain 21/21 for section A because the information is crucial and the learner must know how to handle unexpected situations.  The learner must obtain 7/13 for the knowledge test to be declared competent. The learner must obtain full marks 8/8 for question 5 to be declared competent.

Answers for Evidence Guide

Specific Outcome 5

Reflect on vehicle performance and own operation of vehicle against requirements.

Assessment Criteria

- The effect that weather, road and traffic conditions have on rigid light vehicle performance and driver actions is explained for specific contexts. (Weather conditions include wet weather; road conditions include road surfaces (e.g. dirt roads, gravel roads, potholes, etc.), road shapes (e.g. mountain roads, hill roads, etc.) and road types (e.g., national or regional roads); traffic conditions include high and low density traffic.)

- The influence of driver actions on the cost effective and efficient operation of rigid light vehicle is explained in terms of relevant, specified efficiency and effectiveness criteria.

- The effect of physiological and psychological responses and conditions on driving performance is described in terms of how to manage these responses and conditions. (Psychological responses can include stress; physiological responses can include fatigue.)

| Evidence Required Written Knowledge Test and Report (Reflect on vehicle performance) |  | Evidence sign off |
| --- | --- | --- |
| Section A - List four aspects that may contribute to driver fatigue. (4) The continuous humming sound of driving Warm temperature in the cab Continuous staring at the road Too heavy or large a meal before or during driving - List four things that contribute to economical driving.(4) Correct gear selection Accelerator management Correct tyre pressures Reduced stopping and pulling off Controlled revving and idling - List five aspects that contribute to driver stress.(5) Always in a hurry Excessive driving hours High volumes of traffic Poor road conditions Poor vehicle condition, etc - List three health and medical conditions drivers must monitor.(3) Eye sight Stress levels Epilepsy High or low blood pressure - List four possible symptoms of driver fatigue.(4) Yawning Tired eyes or seeing double Dry mouth Feeling cold (20) Section B Write a report on the vehicle performance and include the following in the report to reflect upon: - (Read Assessment Criteria for more information) Reflection Report: - The weather conditions - Influence of the driver’s actions - Psychological responses and conditions (10) |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

The learner must obtain 15/30 for Section A and B in order to be declared competent.  The assessor must use discretion in Section B.

Answers for Evidence Guide

Specific Outcome 6

Park rigid light vehicle in accordance with specified requirements.

Assessment Criteria

- Vehicle is parked within designated or reserved areas in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Vehicle is shut down in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Vehicle is secured in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Security and convenience factors are taken into account.( Factors include proximity to main pedestrian entrances, access to luggage compartments and ease of conveyance and loading, security of parked and unattended vehicle, etc.)

| Evidence Required Practical Demonstration |  | Evidence sign off |
| --- | --- | --- |
| Task Park rigid light vehicle in accordance with specified requirements The learner must be able to … Practical criteria; Yes; No. 1; Park the vehicle in the correct position. 2; Allow the engine to idle before switching off. 3; Check that the park brake is applied. 4; Check that the lights are off. 5; Record the kilometer reading. 6; Switch off correctly. 7; Stow all items and equipment in the cab out of sight. 8; Clean the cab. 9; Close all windows and lock all doors. 11; Ensure that the canopy is locked, secure & safe. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

The learner must obtain full marks, 11/11 when parking a vehicle in order to comply with Standards.

Unit Standard 2 of this Volume

Answers for Evidence Guide

Unit Standard ID Title:  Operating lift trucks

Unit Standard number:  8038

Specific Outcome 1

Identify and classify freight.

Outcome Notes

Identify and classify freight taking into account documentation, packaging, and labeling associated with the specific freight.

Assessment Criteria

- Explain what he/she is doing and why in selecting specific handling methods and attachments for particular commodities/freight.

- Explain how and why he/she selected a particular piece of lifting equipment for a particular type of freight.

- Explain what you are doing and why in selecting storage facilities for freight.

- Identify the most appropriate course of action in relation to:

  safe working practices and procedures in the handling and storage of freight in general;

  potential hazards in the working environment.

- Explain and demonstrate the operating functions of the particular lift truck relevant for the job environment, giving cognisance to different environmental conditions and freight types.

- Generate and consider options and possibilities for achieving maximum work performance of the equipment through:

  care of lifting equipment and attachments;

  operating of lifting equipment and attachments 
bearing in mind manufacturers` specifications and environmental conditions.

- Evaluate your own performance and identify strengths as well as areas for improvement in terms of the work performance of the equipment.

| Evidence Required Multiple Choice Questions (Make a cross to select the correct answer) |  | Evidence sign off |
| --- | --- | --- |
| 1. The "Load Center" of a truck is measured from the: a) Fork tips, back along the forks. - Pivot point in center of front wheels. - Face or heel of the forks. 2. The lift truck will be at it's most stable when the center of gravity is: - At the top of the mast. - On the left side of the fork carriage. c) Within the stability triangle of the lift truck. 3. When driving on level ground with a load the correct position of mast and forks should be: a) With forks as near to the ground as possible and mast vertical. b) With forks 150-200 mm off the ground and mast vertical. c) With forks 150-200 mm off the ground and mast tilted back. 4. When stacking a load at height, the mast should brought to the vertical position: a) As soon as the load is at the required height. b) When the load is directly over the stack. c) When the load is close to the ground in front of the stack. 5. What is the center of gravity? a) It is the middle of a load. b) It is the place where the object will balance. c) It is the bottom part of a load. Practical demonstration – Checklist The learner is able to … Criteria; Competent; Not yet competent; Remarks. Identify freight taking into account documentation, packaging, and labeling associated with the specific freight.. Classify the freight. Demonstrate quality packaging according to standard procedures. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.

Answers for Evidence Guide

Specific Outcome 2

Handle, load and store freight.

Outcome Notes

Handle, load and store freight in accordance with laid down standards, environmental requirements, and with due consideration to inter alia the commodities and their properties, storage area, placement of load.

Assessment Criteria

- Explain what he/she is doing and why in selecting specific handling methods and attachments for particular commodities/freight.

- Explain how and why he/she selected a particular piece of lifting equipment for a particular type of freight.

- Explain what you are doing and why in selecting storage facilities for freight.

- Explain what you are doing and why in selecting storage facilities for freight.

- Explain and demonstrate the operating functions of the particular lift truck relevant for the job environment, giving cognisance to different environmental conditions and freight types.

- Generate and consider options and possibilities for achieving maximum work performance of the equipment through:

  care of lifting equipment and attachments;

  operating of lifting equipment and attachments 
bearing in mind manufacturers` specifications and environmental conditions.

- Evaluate your own performance and identify strengths as well as areas for improvement in terms of the work performance of the equipment.

| Evidence Required Practical demonstration |  | Evidence sign off |
| --- | --- | --- |
| The learner is able to … Criteria; Competent; Not yet competent; Remarks. Handle, load and store freight in accordance with laid down standards,. Comply with environmental requirements, of the commodities. Store the commodities, in the correct storage area, placement of load.. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.

Answers for Evidence Guide

Specific Outcome 3

Achieve maximum work performance of lifting equipment and attachments.

Outcome Notes

Achieve maximum work performance of lifting equipment and attachments, by applying knowledge of equipment dimensions, controls and capacities, manufacturing specifications and circumstances in the working environment.

Assessment Criteria

- Explain what he/she is doing and why in selecting specific handling methods and attachments for particular commodities/freight.

- Explain what you are doing and why in selecting storage facilities for freight.

- Identify the most appropriate course of action in relation to:

  safe working practices and procedures in the handling and storage of freight in general;

  potential hazards in the working environment.

- Explain and demonstrate the operating functions of the particular lift truck relevant for the job environment, giving cognisance to different environmental conditions and freight types.

- Generate and consider options and possibilities for achieving maximum work performance of the equipment through:

  care of lifting equipment and attachments;

  operating of lifting equipment and attachments 
bearing in mind manufacturers` specifications and environmental conditions.

- Evaluate your own performance and identify strengths as well as areas for improvement in terms of the work performance of the equipment.

# Gap

| Evidence Required (The learner will sit with the assessor and do this assessment on a one-on-one basis. Identify the following lift trucks, symbols and attachments |  | Evidence sign off |
| --- | --- | --- |
| A; B; C. D; E; F. G; H; I J. Did the learner identify each item correctly? ITEM; YES; NO; ITEM; YES; NO. A; F. B; G. C; H. D; I. E; J. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Answers for Evidence Guide

Specific Outcome 4

Operate equipment in accordance with organisational and legislative standards and procedures.

Outcome Notes

Operate equipment in accordance with laid down organisational and legislative standards and procedures as well as manufacturer’s guidelines. "Operating" includes the actual pre-check and operational checks, operating, shutting down and parking of the lift truck.

Assessment Criteria

- Explain what he/she is doing and why in selecting specific handling methods and attachments for particular commodities/freight.

- Explain how and why he/she selected a particular piece of lifting equipment for a particular type of freight.

- Explain what you are doing and why in selecting storage facilities for freight.

- Identify the most appropriate course of action in relation to:

  safe working practices and procedures in the handling and storage of freight in general;

  potential hazards in the working environment.

- Explain and demonstrate the operating functions of the particular lift truck relevant for the job environment, giving cognisance to different environmental conditions and freight types.

- Generate and consider options and possibilities for achieving maximum work performance of the equipment through:

  care of lifting equipment and attachments;

  operating of lifting equipment and attachments 
bearing in mind manufacturers` specifications and environmental conditions.

- Evaluate your own performance and identify strengths as well as areas for improvement in terms of the work performance of the equipment.

Instruction to the Learner

The  learner to perform the pre-start checks and explain the purpose of the components on the lift truck he/she operates daily.

Did the learner check and explain the following?

|  | Components of the lift truck | Checked | Purpose |
| --- | --- | --- | --- |
| 1 | No obstructions under truck |  |  |
| 2 | Condition of attachment |  |  |
| 3 | Position of attachment |  |  |
| 4 | Load back rest |  |  |
| 5 | Carriage |  |  |
| 6 | Mast |  |  |
| 7 | Lifting chains |  |  |
| 8 | Hydraulic lift cylinder |  |  |
| 9 | Hydraulic tilt cylinder |  |  |
| 10 | Overhead guard |  |  |
| 11 | Tyre condition and pressure ( If applicable ) |  |  |
| 12 | Wheel nuts and wheel rims |  |  |
| 13 | Operators compartment clean |  |  |
| 14 | Hydraulic oil level |  |  |
| 15 | Fan belt tension and fan blades |  |  |
| 16 | Water pump |  |  |
| 17 | Counterweight |  |  |
| 18 | Radiator cap |  |  |
| 19 | Radiator water level |  |  |
| 20 | Fuel cap |  |  |
| 21 | Battery electrolyte level |  |  |
| 22 | Battery terminals |  |  |
| 23 | Air filter |  |  |
| 24 | Engine oil level |  |  |
| 25 | Brake fluid level |  |  |
| 26 | All pipes and hoses |  |  |
| 27 | Transmission oil |  |  |
| 28 | Capacity plate |  |  |
| 29 | Any oil or water leaks |  |  |
| 30 | General condition |  |  |
| 31 | Gas cylinder secure ( If applicable ) |  |  |
| 32 | Gas leaks ( If applicable ) |  |  |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.

Answers for Evidence Guide

Specific Outcome 5

Access available support systems and emergency services in case of incidents and accidents.

Assessment Criteria

- Explain what he/she is doing and why in selecting specific handling methods and attachments for particular commodities/freight

- Explain how and why he/she selected a particular piece of lifting equipment for a particular type of freight.

- Explain what you are doing and why in selecting storage facilities for freight.

- Identify the most appropriate course of action in relation to:

  safe working practices and procedures in the handling and storage of freight in general;

  potential hazards in the working environment.

- Explain and demonstrate the operating functions of the particular lift truck relevant for the job environment, giving cognisance to different environmental conditions and freight types.

- Generate and consider options and possibilities for achieving maximum work performance of the equipment through:

  care of lifting equipment and attachments;

  operating of lifting equipment and attachments 
bearing in mind manufacturers` specifications and environmental conditions.

- Evaluate your own performance and identify strengths as well as areas for improvement in terms of the work performance of the equipment.

| Evidence Required Collect evidence of emergency services in case of incidents and accidents |  | Evidence sign off |
| --- | --- | --- |
| Task The learner must collect evidence as mentioned above. Evidence may include but is not limited to emergency situations: - Emergency Plan - Escape route - Fire drills - Signs - Emergency standards regulations Include this in your portfolio of evidence. The assessor will assess your understanding of evidence collected. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.  The learner must be able to identify the access available support systems and emergency services in case of incidents and accidents.  It is crucial to know this because it may be a matter of ‘life’ or ‘death’.

Unit Standard 3 of this Volume

Answers for Evidence Guide

Unit Standard ID Title:  Operate a heavy rigid Vehicle

Unit Standard number:  123253

Specific Outcome 1

Prepare a rigid heavy vehicle for road transport trips according to specification.

Assessment Criteria

- Vital rigid heavy vehicle components are inspected in a systematic way in order to comply with the relevant legislation. (Relevant legislation currently includes the Road Traffic Act of 1993.)

- Rigid heavy vehicle inspection is recorded legibly and in accordance with operational requirements/specifications.

- All rigid heavy vehicle defects are identified and reported accurately, comprehensively and timeously, so that the necessary actions to rectify defects are initiated.

- All prescribed and or other relevant preliminaries are executed as per organisational requirements. (Preliminaries can include brake tests and start up procedures.)

Ask the learner to prepare the vehicle for a trip. Let the learner use an appropriate vehicle inspection list.

Prepare a rigid heavy vehicle for road transport trips according to specification

|  | Specifications | Yes | No |
| --- | --- | --- | --- |
| 1 | Vehicle for any damage |  |  |
| 2 | Vehicle for cleanliness |  |  |
| 3 | The windscreen for damage and cleanliness |  |  |
| 4 | The licence disc is still valid & COF disc |  |  |
| 5 | The windscreen wipers condition and wipe clean |  |  |
| 6 | The head light, indicator lenses & front reflectors |  |  |
| 7 | The registration plate |  |  |
| 8 | The side mirrors |  |  |
| 9 | The doors & windows condition & company logo |  |  |
| 10 | The front right hand side tyre, rim, wheel nuts, valve |  |  |
| 11 | The batteries & cradle |  |  |
| 12 | The air compressor tanks & drain excess condensation |  |  |
| 13 | The visibility tape condition |  |  |
| 14 | The information plate |  |  |
| 15 | The exhaust system |  |  |
| 16 | The condition of the vehicle body |  |  |
| 17 | The spare wheel |  |  |
| 18 | The right hand side rear types, rims, wheel nuts and valves |  |  |
| 19 | The tail & stop light lenses & the rear indicators lenses |  |  |
| 20 | The chevron |  |  |
| 21 | The rear registration plate |  |  |
| 22 | The left hand side rear tyres, rims, wheel nuts and valves |  |  |
| 23 | The fuel tank and cap |  |  |
| 24 | The front left hand side tyre, rim, wheel nuts, valve |  |  |
| 25 | The vehicle attachments & hydraulic condition |  |  |
| 26 | Engine oil level |  |  |
| 27 | Coolant level |  |  |
| 28 | Brake fluid level |  |  |
| 29 | Clutch fluid level |  |  |
| 30 | Power steering fluid level |  |  |
| 31 | Fan belt condition |  |  |
| 32 | All engine components are secure |  |  |
| 33 | The vehicle jack and wheel spanners are present and secure |  |  |
| 34 | The vehicles emergency triangles |  |  |
| 35 | The vehicle fire extinguisher |  |  |
| 36 | Doors & windows operate correctly |  |  |
| 37 | Adjust seat accordingly |  |  |
| 38 | The cab floor is free of loose obstructions |  |  |
| 39 | The rear view mirror |  |  |
| 40 | Vehicle gear selector is in the safe starting position |  |  |
| 41 | Vehicle park brake is applied |  |  |
| 42 | Switch on ignition and check all instruments are working |  |  |
| 43 | The hooter and wipers are working |  |  |
| 44 | The head and tail lights are working |  |  |
| 45 | The indications and hazard lights are working |  |  |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.  The learner must be able to prepare a rigid heavy vehicle for road transport trips according to specification.  It is crucial to know this because it may be a matter of ‘life’ or ‘death’.

Answers for Evidence Guide

Specific Outcome 2

Drive a heavy rigid vehicle in accordance with specified requirements.

Outcome Range

Specified requirements include legal, manufacturer, and defensive driving requirements; current legal requirements include K53.

Assessment Criteria

- Rigid heavy vehicle is driven and maneuvered in accordance with the specified standard Range: K53.

- Rigid heavy vehicle is operated and maneuvered in accordance with manufacturer’s specifications

- Rigid heavy vehicle is driven and maneuvered in accordance with legal provisions (The Road Traffic Act of 1993).

- Rigid heavy vehicle is driven to actively prevent accidents, injury to people or damage to property/vehicle, despite the incorrect actions of others or adverse conditions, by driving safely and in a defensive manner.

The Assessor will explain to the learner what route is required. Make an appointment when you are ready for assessment on the practical demonstration based on the criteria below.

The assessor may ask the learner to drive a specific route according to the K53 system of vehicle control and within the vehicle Manufacturers specifications.

ANY ONE OF THESE  (O) MARKED MEANS THE LEARNER IS NOT YET COMPETENT

|  | USE OF CONTROLS |  | STRENGTHS |  | REMARKS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MANIPULATION OF CONTROLS |  |  |  |  |  |
| 1 |  | Excessive movement |  | ⁮⁮ O |  |  |  |
|  |  | USE OF GEARS |  |  |  |  |  |
| 2 |  | Changes excessively |  | ⁮⁮⁮⁮ O |  |  |  |
| 3 |  | Grates gears |  | ⁮⁮⁮⁮ O |  |  |  |
| 4 |  | Hand rests on top of gear lever |  | ⁮⁮⁮ O |  |  |  |
| 5 |  | Misses or slips a gear |  | O |  |  |  |
|  |  | USE OF CLUTCH |  |  |  |  |  |
| 6 |  | Jerky |  | ⁮⁮⁮ O |  |  |  |
| 7 |  | Slips clutch |  | ⁮ O |  |  |  |
| 8 |  | Rides clutch |  | ⁮⁮ O |  |  |  |
| 9 |  | Late in de-clutching |  | ⁮⁮⁮⁮ O |  |  |  |
| 10 |  | Keeps clutch depressed while stopped |  | ⁮⁮O |  |  |  |
|  |  | USE OF BRAKES |  |  |  |  |  |
| 11 |  | Does not brake progressively |  | ⁮⁮O |  |  |  |
| 12 |  | Park brake not used correctly |  | ⁮⁮ O |  |  |  |
| 13 |  | Brake used unnecessarily (no deceleration) |  | ⁮⁮⁮ O |  |  |  |
| 14 |  | Incorrect use on decline |  | ⁮ O |  |  |  |
| 15 |  | Rests foot on brake pedal |  | ⁮⁮ O |  |  |  |
|  |  | PEDAL BALANCE |  |  |  |  |  |
| 16 |  | Rolling back during pull off |  | ⁮⁮O |  |  |  |
|  |  | STEERING |  |  |  |  |  |
| 17 |  | Crossing arms while turning |  | ⁮⁮⁮⁮ O |  |  |  |
| 18 |  | Hands on steering wheel position |  | ⁮⁮⁮ O |  |  |  |
| 19 |  | Steers with one hand |  | ⁮ O |  |  |  |
|  |  | ACCELERATOR MANAGEMENT |  |  |  |  |  |
| 20 |  | Does not maintain constant speed |  | ⁮⁮ O |  |  |  |
| 21 |  | Blipping |  | ⁮⁮ O |  |  |  |
| 22 |  | Lets engine labour |  | ⁮⁮ O |  |  |  |
| 23 |  | Over-revs |  | ⁮⁮ O |  |  |  |
|  |  | USE OF RETARDATION DEVICE |  |  |  |  |  |
| 24 |  | Not according to specification |  | ⁮⁮ O |  |  |  |
|  |  | COASTING |  |  |  |  |  |
| 25 |  | De-clutch too early |  | ⁮⁮⁮ O |  |  |  |
| 26 |  | Select neutral too early |  | ⁮⁮ O |  |  |  |
|  |  | ROAD SENSE |  |  |  |  |  |
|  |  | ROAD OBSERVATION |  |  |  |  |  |
| 27 |  | No observation |  | ⁮⁮ O |  |  |  |
| 28 |  | Ignores changes in road surface |  | ⁮⁮⁮ O |  |  |  |
| 29 |  | Ignores road signs & markings |  | ⁮⁮ O |  |  |  |
| 30 |  | Moves into other vehicles blind spots |  | ⁮⁮ O |  |  |  |
| 31 |  | Incorrect use of mirrors |  | ⁮⁮⁮ O |  |  |  |
|  |  | VEHICLE POSITIONING |  |  |  |  |  |
| 32 |  | Travels to close to vehicle in front |  | ⁮⁮ O |  |  |  |
| 33 |  | Travel too far from vehicle in front |  | ⁮⁮⁮ O |  |  |  |
| 34 |  | Travels too far over in lane |  | ⁮⁮⁮⁮ O |  |  |  |
|  |  | SPEED |  |  |  |  |  |
| 35 |  | Ignores speed limit |  | ⁮ O |  |  |  |
| 36 |  | Takes too long to pull off at traffic lights, etc. |  |  |  |  |  |
|  |  | ASSESSMENT OF HAZARDS |  |  |  |  |  |
| 37 |  | Fails to notice hazard |  | ⁮⁮ O |  |  |  |
| 38 |  | Fails to react to hazard |  | ⁮⁮ O |  |  |  |
|  |  | CORNERING |  |  |  |  |  |
| 39 |  | Cuts corner |  | ⁮⁮ O |  |  |  |
| 40 |  | Too fast |  | ⁮ O |  |  |  |
| 41 |  | Mounts curb |  | ⁮ O |  |  |  |
| 42 |  | Too wide |  | ⁮ O |  |  |  |
|  |  | OVERTAKING |  |  |  |  |  |
| 43 |  | Fails to check safety ahead |  | O |  |  |  |
| 44 |  | Fails to check safety behind |  | O |  |  |  |
| 45 |  | Fails to back down |  | ⁮ O |  |  |  |
| 46 |  | Accelerates when being overtaken |  | ⁮ O |  |  |  |
|  |  | COMMUNICATION AND ATTITUDE |  |  |  |  |  |
| 47 |  | Hooter used when not required |  | ⁮ O |  |  |  |
| 48 |  | Indicators not used correctly |  | ⁮⁮⁮ O |  |  |  |
| 49 |  | Shows irritation of other road users |  | ⁮ O |  |  |  |
| 50 |  | Demonstrates displeasure to others |  | O |  |  |  |
|  |  | MANOEUVERING |  |  |  |  |  |
| 51 |  | Alley docking from left |  | ⁮⁮⁮⁮ O |  |  |  |
| 52 |  | Alley docking from right |  | ⁮⁮⁮⁮ O |  |  |  |
| 53 |  | Reversing in a straight line |  | ⁮⁮⁮ O |  |  |  |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.  The learner must be able Drive a heavy rigid vehicle in accordance with specified requirements. It is crucial to know this because it may be a matter of ‘life’ or ‘death’.

Answers for Evidence Guide

Specific Outcome 3

Ensure the maintenance of road transport service quality.

Assessment Criteria

- Time schedules are considered, implications for freight/passengers are established, and appropriate action is taken to ensure the maintenance of freight quality/passenger safety and comfort.

- The impact of load positioning on the stability of the rigid heavy vehicle is accurately described.

- Driving style is adapted in order to maintain freight quality/passenger safety and comfort during transportation.

- Organisational standards are adhered to with full consideration of safety and comfort factors and actions implemented to rectify problems are appropriate for specific contexts. (Safety factors can include stopping only at scheduled locations, passenger door only opened and closed when completely stationary, load (e.g., number of passengers, freight mass, loading of coach and trailer, etc.) restricted to design and legal limit of vehicle, luggage and emergency doors fully secured, attachment of luggage trailer confirmed; Comfort factors include acceleration and deceleration forces applied gradually; cornering speeds maintained within comfort levels; moving off only when passengers are seated or standing securely.)

| Evidence Required |  | Evidence sign off |
| --- | --- | --- |
| The learner must include the following evidence in the Portfolio of Evidence. Criteria; Managing unforeseen problems; Yes; No. Time schedules; The learner communicated any implications relating to freight or passengers. The learner has taken appropriate action to the maintenance of fright quality and or ensured the passengers safety in line with the safety procedures. The learner is able to describe the impact of load positioning on the stability of the rigid light vehicle; The learner is able to illustrate how to resolve a problem when the freight is not stable. Practical Demonstration: The learner is able to demonstrate …; Managing unforeseen problems; Yes; No. That the driving style is adapted in order to maintain freight handling during transportation; The learner is able to demonstrate how he/she will act if there are unforeseen problems e.g. weather, rain etc.. Organisational standards are adhered to with full consideration of safety and comfort factors and appropriate actions implemented to rectify problems are appropriate for specific contexts. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Assessment Criteria for Competency:

In order to declare the learner competent he/she must meet all the specifications in the practical demonstration assessment.  The assessor must use his/her own discretion based on his/her practical experience.

Answers for Evidence Guide

Specific Outcome 4

Handle unexpected situations according to specified procedures.

Outcome Range

Unexpected situations can include, but are not limited to incidents, accidents, breakdowns, fire on vehicle, medical emergency, hi-jacking, mechanical failure (e.g. burst tyre), spillage/load loss, etc.

Assessment Criteria

- Rigid heavy vehicle and cab instruments are monitored for malfunctions and appropriate action is taken, where required.

- Driving is adapted to unexpected situations safely and according to specified procedures.

- Unexpected situations are reported according to operational procedures and legal requirements.

- Description of corrective actions to get freight/passengers to their destinations in the event of route deviations, delays experienced and breakdowns, as well as the company guidelines for consideration in selecting course of action are appropriate for specific contexts.

| Evidence Required Practical Questioning and Illustration The learner must explain how he/she will adapt to the following situations |  | Evidence sign off |
| --- | --- | --- |
| Section A: Practical Practical Questioning and Illustration (Strictly use the Health and Safety Procedures and other standards in this context) The learner must be able to demonstrate action and explain the steps to follow with the following unforeseen problems that may occur: (Mark a tick next to each answer that was provided and where learner is found competent) - Accidents Fire on vehicle - Medical emergency Hi-jacking - Mechanical failure - Spillage Potholes - Bad weather conditions 21 Marks Written Knowledge Test Question 1 (1) What must a hijacker do to hijack your vehicle? Must stop it and be close enough to get to the driver or passenger Question 2 (6) What should a driver do if he / she is hijacked? - Stay calm - Do what they say and give them what they ask for - Do not put your hands up unless they say so - Do not put your hands up unless they say so - Do not move suddenly or try to get something from your pocket unless they say you must - Call for assistance as soon as possible after the hijack Question 3 (3) List three additional aspects a driver must consider apart from driving the vehicle. - Road conditions - Traffic conditions - Driver fatigue Question 4 (3) List three possible causes of a skid. - Excessive speed - Harsh braking - Harsh acceleration Total: 13 Problem solving question (In line with company procedures) (8) Question 5 Explain the procedures to follow when you are involved in an accident where a pedestrian has been killed … The learner stated the following criteria: Criteria; Yes; No. 1; Stop the vehicle immediately where it is?. 2; Make the accident scene safe to prevent other accidents?. 3; Ask someone to call for help?. 4; Render any applicable first aid that he/she is trained for?. 5; Take down victims and witnesses details?. 6; Draw a sketch of the scene?. 7; Report the accident to the police within 24 hours?. 8; Apply the company specific procedures?. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

The learner must obtain 21/21 for section A because the information is crucial and the learner must know how to handle unexpected situations.  The learner must obtain 7/13 for the knowledge test to be declared competent.  The learner must obtain full marks 8/8 for question 5 to be declared competent.

Answers for Evidence Guide

Specific Outcome 5

Park rigid heavy vehicle in accordance with specified requirements.

Assessment Criteria

- Rigid heavy vehicle is parked within designated or reserved areas in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Rigid heavy vehicle is shut down in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Rigid heavy vehicle is secured in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Security and convenience factors are taken into account. (Factors include proximity to main pedestrian entrances, access to luggage compartments and ease of conveyance and loading, security of parked and unattended vehicle, etc.)

| Evidence Required Practical Demonstration |  | Evidence sign off |
| --- | --- | --- |
| Task Park rigid heavy vehicle in accordance with specified requirements The learner must be able to … Practical criteria; Yes; No. 1; Park the vehicle in the correct position. 2; Allow the engine to idle before switching off. 3; Check that the park brake is applied. 4; Check that the lights are off. 5; Record the kilometer reading. 6; Switch off correctly. 7; Stow all items and equipment in the cab out of sight. 8; Clean the cab. 9; Close all windows and lock all doors. 11; Ensure that the canopy is locked, secure & safe. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Assessment Criteria for Competency:

The learner must obtain full marks, 11/11 when parking a vehicle in order to comply with Standards.

Unit Standard 4 of this Volume

Answers for Evidence Guide

Unit Standard ID Title:  Operate a Vehicle Combination

Unit Standard number:  123254

Specific Outcome 1

Prepare a vehicle combination for road transport trips according to specification.

Assessment Criteria

- Vital vehicle combination components are inspected in a systematic way in order to comply with the relevant legislation.( Relevant legislation currently includes the Road Traffic Act of 1993.)

- Rigid heavy vehicle inspection is recorded legibly and in accordance with operational requirements/specifications.

- Vehicle combination defects are identified and reported accurately, comprehensively and timeously, so that the necessary actions to rectify defects are initiated.

- All prescribed and or other relevant preliminaries are executed.

- (Preliminaries can include brake tests and start up procedures.)

Ask the learner to prepare the vehicle combination for a trip. Let the learner use an appropriate vehicle inspection list.

|  | Vehicle combinations criteria | Yes | No |
| --- | --- | --- | --- |
| 1 | Vehicle for any damage |  |  |
| 2 | Vehicle for cleanliness |  |  |
| 3 | The windscreen for damage and cleanliness |  |  |
| 4 | The drawing vehicle license disc is still valid & COF disc |  |  |
| 5 | The windscreen wipers condition and wipe clean |  |  |
| 6 | The head light, indicator lenses & front reflectors |  |  |
| 7 | The registration plate |  |  |
| 8 | The side mirrors |  |  |
| 9 | The doors & windows condition & company logo |  |  |
| 10 | The drawing vehicles front right hand side tyre, rim, wheel nuts, valve |  |  |
| 11 | The batteries & cradle |  |  |
| 12 | The air compressor tanks & drain excess condensation |  |  |
| 13 | The drawing vehicles visibility tape condition |  |  |
| 14 | The drawing vehicles information plate |  |  |
| 15 | The exhaust system |  |  |
| 16 | The condition of the vehicle body |  |  |
| 17 | The drawing vehicles spare wheel |  |  |
| 18 | The drawing vehicles right hand side rear types, rims, wheel nuts and valves |  |  |
| 19 | The drawing vehicles tail & stop light lenses & the rear indicators lenses |  |  |
| 20 | The drawing vehicles chevron |  |  |
| 21 | The drawing vehicles rear registration plate |  |  |
| 22 | The drawing vehicles left hand side rear tyres, rims, wheel nuts and valves |  |  |
| 23 | The fuel tank and cap |  |  |
| 24 | The drawing vehicles front left hand side tyre, rim, wheel nuts, valve |  |  |
| 25 | The vehicle attachments & hydraulic condition |  |  |
| 26 | Engine oil level |  |  |
| 27 | Coolant level |  |  |
| 28 | Brake fluid level |  |  |
| 29 | Clutch fluid level |  |  |
| 30 | Power steering fluid level |  |  |
| 31 | Fan belt condition |  |  |
| 32 | All engine components are secure |  |  |
| 33 | The vehicle jack and wheel spanners are present and secure |  |  |
| 34 | The vehicles emergency triangles |  |  |
| 35 | The vehicle fire extinguisher |  |  |
| 36 | Doors & windows operate correctly |  |  |
| 37 | Adjust seat accordingly |  |  |
| 38 | The cab floor is free of loose obstructions |  |  |
| 39 | The rear view mirror |  |  |
| 40 | Vehicle gear selector is in the safe starting position |  |  |
| 41 | Drawing vehicles park brake is applied |  |  |
| 42 | Switch on ignition and check all instruments are working |  |  |
| 43 | The hooter and wipers are working |  |  |
| 44 | The head and tail lights are working |  |  |
| 45 | The drawing vehicles indications and hazard lights are working |  |  |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.  The learner must be able to prepare a vehicle combination for road transport trips according to specification.  It is crucial to know this because it may be a matter of ‘life’ or ‘death’.

Answers for Evidence Guide

Specific Outcome 2

Drive a vehicle combination in accordance with specified requirements.

Outcome Range

Specified requirements include legal, manufacturer, and defensive driving requirements; current legal requirements include K53.

Assessment Criteria

- Vehicle combination is driven and manoeuvred in accordance with the specified standard.

- Vehicle combination is driven in accordance with manufacturer`s specifications and to actively prevent accidents, injury to people or damage to property/vehicle, despite the incorrect actions of others or adverse conditions, by applying defensive driving principles. (The Road Traffic Act of 1993.)

- Vehicle combination is driven in a manner, which takes cognisance of the articulation point/s.

The Assessor will explain to the learner what route is required. Make an appointment when you are ready for assessment on the practical demonstration based on the criteria below.

The assessor may ask the learner to drive a specific route according to the K53 system of vehicle control and within the vehicle Manufacturers specifications.

ANY ONE OF THESE  (O) MARKED MEANS THE LEARNER IS NOT YET COMPETENT

|  | USE OF CONTROLS |  | STRENGTHS |  | REMARKS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MANIPULATION OF CONTROLS |  |  |  |  |  |
| 1 |  | Excessive movement |  | ⁮⁮ O |  |  |  |
|  |  | USE OF GEARS |  |  |  |  |  |
| 2 |  | Changes excessively |  | ⁮⁮⁮⁮ O |  |  |  |
| 3 |  | Grates gears |  | ⁮⁮⁮⁮ O |  |  |  |
| 4 |  | Hand rests on top of gear lever |  | ⁮⁮⁮ O |  |  |  |
| 5 |  | Misses or slips a gear |  | O |  |  |  |
|  |  | USE OF CLUTCH |  |  |  |  |  |
| 6 |  | Jerky |  | ⁮⁮⁮ O |  |  |  |
| 7 |  | Slips clutch |  | ⁮ O |  |  |  |
| 8 |  | Rides clutch |  | ⁮⁮ O |  |  |  |
| 9 |  | Late in de-clutching |  | ⁮⁮⁮⁮ O |  |  |  |
| 10 |  | Keeps clutch depressed while stopped |  | ⁮⁮O |  |  |  |
|  |  | USE OF BRAKES |  |  |  |  |  |
| 11 |  | Does not brake progressively |  | ⁮⁮O |  |  |  |
| 12 |  | Park brake not used correctly |  | ⁮⁮ O |  |  |  |
| 13 |  | Brake used unnecessarily (no deceleration) |  | ⁮⁮⁮ O |  |  |  |
| 14 |  | Incorrect use on decline |  | ⁮ O |  |  |  |
| 15 |  | Rests foot on brake pedal |  | ⁮⁮ O |  |  |  |
|  |  | PEDAL BALANCE |  |  |  |  |  |
| 16 |  | Rolling back during pull off |  | ⁮⁮O |  |  |  |
|  |  | STEERING |  |  |  |  |  |
| 17 |  | Crossing arms while turning |  | ⁮⁮⁮⁮ O |  |  |  |
| 18 |  | Hands on steering wheel position |  | ⁮⁮⁮ O |  |  |  |
| 19 |  | Steers with one hand |  | ⁮ O |  |  |  |
|  |  | ACCELERATOR MANAGEMENT |  |  |  |  |  |
| 20 |  | Does not maintain constant speed |  | ⁮⁮ O |  |  |  |
| 21 |  | Blipping |  | ⁮⁮ O |  |  |  |
| 22 |  | Lets engine labour |  | ⁮⁮ O |  |  |  |
| 23 |  | Over-revs |  | ⁮⁮ O |  |  |  |
|  |  | USE OF RETARDATION DEVICE |  |  |  |  |  |
| 24 |  | Not according to specification |  | ⁮⁮ O |  |  |  |
|  |  | COASTING |  |  |  |  |  |
| 25 |  | De-clutch too early |  | ⁮⁮⁮ O |  |  |  |
| 26 |  | Select neutral too early |  | ⁮⁮ O |  |  |  |
|  |  | ROAD SENSE |  |  |  |  |  |
|  |  | ROAD OBSERVATION |  |  |  |  |  |
| 27 |  | No observation |  | ⁮⁮ O |  |  |  |
| 28 |  | Ignores changes in road surface |  | ⁮⁮⁮ O |  |  |  |
| 29 |  | Ignores road signs & markings |  | ⁮⁮ O |  |  |  |
| 30 |  | Moves into other vehicles blind spots |  | ⁮⁮ O |  |  |  |
| 31 |  | Incorrect use of mirrors |  | ⁮⁮⁮ O |  |  |  |
|  |  | VEHICLE POSITIONING |  |  |  |  |  |
| 32 |  | Travels to close to vehicle in front |  | ⁮⁮ O |  |  |  |
| 33 |  | Travel too far from vehicle in front |  | ⁮⁮⁮ O |  |  |  |
| 34 |  | Travels too far over in lane |  | ⁮⁮⁮⁮ O |  |  |  |
|  |  | SPEED |  |  |  |  |  |
| 35 |  | Ignores speed limit |  | ⁮ O |  |  |  |
| 36 |  | Takes too long to pull off at traffic lights, etc. |  |  |  |  |  |
|  |  | ASSESSMENT OF HAZARDS |  |  |  |  |  |
| 37 |  | Fails to notice hazard |  | ⁮⁮ O |  |  |  |
| 38 |  | Fails to react to hazard |  | ⁮⁮ O |  |  |  |
|  |  | CORNERING |  |  |  |  |  |
| 39 |  | Cuts corner |  | ⁮⁮ O |  |  |  |
| 40 |  | Too fast |  | ⁮ O |  |  |  |
| 41 |  | Mounts curb |  | ⁮ O |  |  |  |
| 42 |  | Too wide |  | ⁮ O |  |  |  |
|  |  | OVERTAKING |  |  |  |  |  |
| 43 |  | Fails to check safety ahead |  | O |  |  |  |
| 44 |  | Fails to check safety behind |  | O |  |  |  |
| 45 |  | Fails to back down |  | ⁮ O |  |  |  |
| 46 |  | Accelerates when being overtaken |  | ⁮ O |  |  |  |
|  |  | COMMUNICATION AND ATTITUDE |  |  |  |  |  |
| 47 |  | Hooter used when not required |  | ⁮ O |  |  |  |
| 48 |  | Indicators not used correctly |  | ⁮⁮⁮ O |  |  |  |
| 49 |  | Shows irritation of other road users |  | ⁮ O |  |  |  |
| 50 |  | Demonstrates displeasure to others |  | O |  |  |  |
|  |  | MANOEUVERING |  |  |  |  |  |
| 51 |  | Alley docking from left |  | ⁮⁮⁮⁮ O |  |  |  |
| 52 |  | Alley docking from right |  | ⁮⁮⁮⁮ O |  |  |  |
| 53 |  | Reversing in a straight line |  | ⁮⁮⁮ O |  |  |  |

Assessment Criteria for Competency:

The learner must obtain full marks for this section in order to be declared competent.  The learner must be able to drive a vehicle combination in accordance with specified requirements. It is crucial to know this because it may be a matter of ‘life’ or ‘death’.

Answers for Evidence Guide

Specific Outcome 3

Ensure the maintenance of road transport service quality.

Assessment Criteria

- Time schedules are considered, implications for freight/passengers are established, and appropriate action is taken to ensure the maintenance of freight quality/passenger safety and comfort.

- The impact of load positioning on the stability of the vehicle combination is accurately described.

- The impact of additional trailers on the dynamics of the vehicle combination is explained in terms of the interaction of the following aspects and their impact on the stability of the vehicle combination:

  Mass of individual vehicle units in the combination.

  Speed of the combination.

  Direction of travel.

  Road surface.

  Load distribution.

  Jack-knifing.

- Driving style is adapted in order to maintain freight quality/passenger safety and comfort during transportation

- Organisational standards are adhered to with full consideration of safety and comfort factors and actions are implemented to rectify problems for specific contexts. (Safety factors can include stopping only at scheduled locations, passenger door only opened and closed when completely stationary, load (e.g., number of passengers, freight mass, loading of coach and trailer, etc.) restricted to design and legal limit of vehicle, luggage and emergency doors fully secured, attachment of luggage trailer confirmed; Comfort factors include acceleration and deceleration forces applied gradually; cornering speeds maintained within comfort levels; moving off only when passengers are seated or standing securely.)

| Evidence Required Practical Illustration |  | Evidence sign off |
| --- | --- | --- |
| The learner must include the following evidence in the Portfolio of Evidence. Criteria; Managing unforeseen problems; Yes; No. Time schedules; The learner communicated any implications relating to freight or passengers. The learner has taken appropriate action to the maintenance of fright quality and or ensured the passengers safety in line with the safety procedures. The learner is able to describe the impact of load positioning on the stability of the rigid light vehicle; The learner is able to illustrate how to resolve a problem when the freight is not stable. Practical Demonstration: The learner is able to demonstrate …; Managing unforeseen problems; Yes; No. That the driving style is adapted in order to maintain freight handling during transportation; The learner is able to demonstrate how he/she will act if there are unforeseen problems e.g. weather, rain etc.. Organisational standards are adhered to with full consideration of safety and comfort factors and appropriate actions implemented to rectify problems are appropriate for specific contexts. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Assessment Criteria for Competency:

In order to declare the learner competent he/she must meet all the specifications in the practical demonstration assessment.  The assessor must use his/her own discretion based on his/her practical experience.

Answers for Evidence Guide

Specific Outcome 4

Handle unexpected situations according to specified procedures.

Outcome Range:

Unexpected situations can include, but are not limited to incidents, accidents, breakdowns, fire on vehicle, medical emergency, hi-jacking, mechanical failure (e.g. burst tyre), spillage/load loss, etc.

Assessment Criteria

- Vehicle and cab instruments are monitored for malfunctions and appropriate action is taken, where required.

- Driving is adapted to unexpected situations safely and according to specified procedures.

- Unexpected situations are reported according to operational procedures and legal requirements.

| Evidence Required Practical Questioning and Illustration The learner must explain how he/she will adapt to the following situations |  | Evidence sign off |
| --- | --- | --- |
| Section A: Practical Practical Questioning and Illustration (Strictly use the Health and Safety Procedures and other standards in this context) The learner must be able to demonstrate action and explain the steps to follow with the following unforeseen problems that may occur: (Mark a tick next to each answer that was provided and where learner is found competent) - Accidents Fire on vehicle - Medical emergency Hi-jacking - Mechanical failure - Spillage Potholes - Bad weather conditions 21 Marks Written Knowledge Test Question 1 (1) What must a hijacker do to hijack your vehicle? Must stop it and be close enough to get to the driver or passenger Question 2 (6) What should a driver do if he / she is hijacked? - Stay calm - Do what they say and give them what they ask for - Do not put your hands up unless they say so - Do not put your hands up unless they say so - Do not move suddenly or try to get something from your pocket unless they say you must - Call for assistance as soon as possible after the hijack Question 3 (3) List three additional aspects a driver must consider apart from driving the vehicle. - Road conditions - Traffic conditions - Driver fatigue Question 4 (3) List three possible causes of a skid. - Excessive speed - Harsh braking - Harsh acceleration Total: 13 Problem solving question (In line with company procedures) (8) Question 5 Explain the procedures to follow when you are involved in an accident where a pedestrian has been killed … The learner stated the following criteria: Criteria; Yes; No. 1; Stop the vehicle immediately where it is?. 2; Make the accident scene safe to prevent other accidents?. 3; Ask someone to call for help?. 4; Render any applicable first aid that he/she is trained for?. 5; Take down victims and witnesses details?. 6; Draw a sketch of the scene?. 7; Report the accident to the police within 24 hours?. 8; Apply the company specific procedures?. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |
|  |  | Date |

Assessment Criteria for Competency:

The learner must obtain 21/21 for section A because the information is crucial and the learner must know how to handle unexpected situations.  The learner must obtain 7/13 for the knowledge test to be declared competent.  The learner must obtain full marks 8/8 for question 5 to be declared competent.

Answers for Evidence Guide

Specific Outcome 5

Park vehicle combination in accordance with specified requirements.

Assessment Criteria

- Vehicle combination is parked within designated or reserved areas in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Vehicle combination is shut down in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Vehicle combination is secured in accordance with manufacturer specifications, traffic regulations and operational procedures.

- Security and convenience factors are taken into account.( Factors include proximity to main pedestrian entrances, access to luggage compartments and ease of conveyance and loading, security of parked and unattended vehicle, etc.)

| Evidence Required |  | Evidence sign off |
| --- | --- | --- |
| Task Park rigid heavy vehicle in accordance with specified requirements The learner must be able to … Practical criteria; Yes; No. 1; Park the vehicle in the correct position. 2; Allow the engine to idle before switching off. 3; Check that the park brake is applied. 4; Check that the lights are off. 5; Record the kilometer reading. 6; Switch off correctly. 7; Stow all items and equipment in the cab out of sight. 8; Clean the cab. 9; Close all windows and lock all doors. 11; Ensure that the canopy is locked, secure & safe. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Assessment Criteria for Competency:

The learner must obtain full marks, 11/11 when parking a vehicle in order to comply with Standards.

Answers for Evidence Guide

Specific Outcome 6

Couple and uncouple a drawing vehicle and trailer/s according to specified procedures.

Assessment Criteria

- Procedures for coupling and uncoupling a drawing vehicle and trailer/s are appropriate for specific trailer type.

- All systems connecting trailer to drawing vehicle are functional when coupling

- Systems include suzi-pipes, electrical connections, fifth wheel, and locking pin.

- All systems are correctly disconnected, when uncoupling.

| Evidence Required Practical |  | Evidence sign off |
| --- | --- | --- |
| Trailer Checklist Trailer criteria; Yes; No. 1; Trailer for any damage. 2; Trailer for cleanliness. 3; The trailers licence disc is still valid & COF disc. 4; The trailers indicator lenses & reflectors. 5; The trailers registration plate. 6; The trailers right hand side tyres, rims, wheel nuts, valves. 7; The trailers visibility tape condition. 8; The trailers information plate. 9; The trailers spare wheel/s. 10; The trailers tail & stop light lenses. 11; The trailers park brake is applied. 12; The trailers landing legs. 13; The trailers chevron. 14; The trailers left hand side rear tyres, rims, wheel nuts and valves. 15; The trailers attachments & hydraulic condition. |  | Self-assessment |
|  |  | Initial |
|  |  | Date |
|  |  | ECF evaluation |
|  |  | Initial |

Assessment Criteria for Competency

The learner must be able to couple and uncouple a drawing vehicle and trailer/s according to specified procedures.   The learner must obtain full marks.